TEACHER:MWALE CH

DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC:	TRIANGLES	SUBTOPIC: TRIANGLES

LEARNING OUTCOMES LSBAT

- To construct a triangle to satisfy given conditions

RATIONALE

Knowledge:	Construction of scalene triangles using sides

Skills;	Accurate construction of triangles

Values:	Awareness of methods of constructing triangle

TEACHING AIDS

……………………………………………………………………………………………………………………………………………

| PART | LESSON CONTENT | METHODOLO GY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introduction | - Recap on the previous lesson | Questions and answers | Provide answers |
| Developme nt | TRIANGLES TO CONSTRUCT A TRIANGLE TO SATISFY GIVEN CONDITIONS EXAMPLE ONE Construct the scalene triangle given that AB= 85mm, AC= 95mm and BC= 70 mm. - Draw a base line AB= 85mm - At centre A, radius AC=95mm construct an arc above as shown. - At centre A, radius BC=70mm construct an arc to cross with the first arc at point C as shown. - Join ABC using bold lines to give the required scalene triangle. EXAMPLE TWO To construct a scalene triangle when two angles as 730 and 540 and the included side of 60mm - Draw a line and mark off AB= 60 on it. - At point construct angle 730 at A and 54 at B - The two lines meet at C. |  |  |
| Application | CLASS EXERCISE - Construct the following scalene triangle - Given that AB= 89mm, AC= 80 mm and BC= 75 mm - Given that AB= 90mm, AC=110, CAB= 420 - Given that AB=BC=AC=68 - Right angle triangle given that AB= 60, BC=80, AC=100 | Questions And Answers | Provide answers |
| Conclusion | - Recap on the main steps and points | V/E |  |

LESSON EVALUATION

TEACHER:MWALE CH

DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC:	TRIANGLES	SUBTOPIC: TRIANGLES

LEARNING OUTCOMES LSBAT

- Construct  an isosceles triangle given the perimeter and the altitude

RATIONALE

Knowledge:	Construction of scalene triangles using sides

Skills;	Accurate construction of triangles

Values:	Awareness of methods of constructing triangle

TEACHING AIDS

| PART | LESSON CONTENT | METHODO LOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introduct ion | - Recap on the previous lesson | Questions and answers | Provide answers |
| Develop ment | TRIANGLES EXAMPLE ONE Construct an isosceles triangle given the perimeter 180mm and the altitude 60mm - Draw line AB equal to half the perimeter. - From B erect a perpendicular and make BC equal to the altitude. - Join AC and bisect it to cut AB in D. - Produce DB so that BE= BD. CDE is the required triangle EXAMPLE TWO To construct a triangle given the perimeter 120mm and the ratio 4:3:6 Draw the line AB equal in length to the perimeter. - Divide AB into the required ratio 4:3:6. - With centre C and radius CA, draw an arc. - With centre D and radius DB, draw an arc to intersect the first arc in E. - ECD is the required triangle |  |  |
| Applicati on | CLASS EXERCISE - Construct an isosceles triangle given the perimeter 180mm and the altitude 60mm - To construct a triangle given the perimeter 120mm and the ratio 4:3:5 | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |

LESSON EVALUATION

# HOME WORK/ EXERCISE

TOPIC: TRIANGLES	CLASS:…………………………..	DATE:…………………………..

- Construct an equilateral triangle with sides 60 mm long.

- Construct an isosceles triangle that has a perimeter of 135 mm and an altitude of 55 mm.

- Construct a triangle with base angles 60 ° and 45 ° and an altitude of 76 mm.

- Construct a triangle with a base of 55  mm,  an  altitude  of  62  mm  and  a  vertical  angle  of 37.50

- Construct a triangle with a perimeter measuring 160 mm and sides in the ratio 3:5:6.

- Construct a triangle with a perimeter of 170 mm and sides in the ratio 7:3:5.

- Construct a triangle given that the perimeter is 115 mm, the altitude is 40 mm and the  vertical angle is 45 °

- Construct a triangle with  a  base  measuring  62  mm,  an  altitude  of  50  mm  and  a  vertical  angle of  60 ° . Now draw a similar triangle with a perimeter of 250 mm.

- Construct a triangle with a perimeter of 125 mm, whose sides are in the ratio 2:4:5.