TEACHER:MWALE CHIPO

# DESIGN AND TECHNOLOGY DEPARTMENT

### LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC: FORM WORK	SUBTOPIC: FORM WORK FOR SLAB LEARNING OUTCOMES LSBAT

- Define form work

- Identify three types of frame work

- Identify the recommended timber to use in construction (shuttering)

- Erect form work for concrete walls

### RATIONALE

Knowledge:	Work plan, Material selection, Form work positions, Construct formwork

Skills:	Interpretation of, work plans, Constructing, form work

Values:

- Awareness of assembling parts of a hipped and valley roof and Observation of safety precautions when constructing roofs and Appreciation of beauty when finishing construction work

TEACHING AIDS

| PART | LESSON CONTENT | METHODOLO GY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introduction | - Recap on the previous lesson | Questions and answers | Provide answers |
| Development | FORM WORK - Form work is the term given to either temporary or permanent moulds into which concrete or similar materials are poured. - Shuttering refer to where planks are fixed together to hold concrete in order to construct a lintel or pillar. Types of frame work - Concrete slab frame work - Construction form work - Concrete mix design RECOMMENDED TIMBER TO USE IN FORMWORK( SHUTTERING) - Well-seasoned (dried) to avoid complications such as cracking - Light weight - Easy to nail without cracking - Free from defects (warping) because these will transfer to the concrete - Smooth on both sides to give the concrete a clean finish. REQUIRED SIZES FOR SLAB CONSTRUCTION | Questions And Answers Demonstration V/E | Taking note of important points Provide answers |
| Application | CLASS EXERCISE - Clearly indicate the advantages and disadvantages of using untreated timber for shuttering | Questions And Answers | Provide answers |
| Conclusion | - Recap on the main steps and points | V/E |  |

| Description | Size (mm) |
| --- | --- |
| Beam columns for sides and bottom | 25mm-40mm thick |
| Joints or edge | 50 x 70 to 50 x 150 mm |
| Posts for structure support | 75 x 100 to 100 x 100 |
| Plywood | Standard thickness |

TEACHER EVALUATION

TEACHER:MWALE CHIPO

### DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… …  DATE:……/……./20….     DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC: FORM WORK	SUBTOPIC: ROOF CONSTRUCTION LEARNING OUTCOMES LSBAT

- Identify parts of a hip roof

- Interpret plans for a hipped roof

- Construct a hipped and valleys roof

### RATIONALE

Knowledge:        Roof terminologies/ Parts of a hipped and valley roof/

Skills:	Identification of parts of a hipped roof /Interpretation of roof drawings/ Application of principles

Values: Observation of safety precautions when constructing roofs and

### TEACHING AIDS

| PART | LESSON CONTENT | METHODOL OGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introduction | - Recap on the previous lesson | Questions and answers | Provide answers |
| Development | ROOF CONSTRUCTION HIPPED ROOF - A hipped roof is a type of roof that considered rafters covered in all the corners leaving no naked wall exposed to external weather conditions SAFETY WHEN WORKING ON ROOFS - Erecting scaffolding up to lower level of the roof to save anyone who slips down a roof slope - Pick a clear, calm, cool time of day to work on roofs. Wet roofs are slippery. Wind also poses a danger, and excessive heat softens the shingles, making them vulnerable to damage. - Wear shoes with a soft rubber sole for extra traction. - Keep the bottom of your shoes free of mud and dirt, and the roof swept clear of dirt and debris - Always watch out for fall hazards when working at heights. PARTS OF A HIPPED ROOF |  |  |
| Application | CLASS EXERCISE Draw and label the parts of a truss | Questions And Answers | Provide answers |
| Conclusion | - Recap on the main steps and points | V/E |  |

LESSON EVALUATION