TEACHER:MWALE CH

## DESIGN AND TECHNOLOGY DEPARTMENT

#### LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20…. DURATION:………….

## BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC: MANUFACTURING MATERIALS	SUBTOPIC: FINISHING PROCESS

## LEARNING OUTCOMES LSBAT

### Define finishing

- Describe abrasives materials

- State safety precautions when using abrasive.

RATIONALE

Knowledge:	Types of abrasives:  Glass paper, emery cloth, Grades of abrasives,

Skills:	Identification of abrasives. Application of abrasives and abrading techniques

Values:	Appreciation of use of abrasives in finishing artifacts.

Awareness of different types of abrasives Observing safety precautions when using abrasives

TEACHING AIDS

……………………………………………………………………………………………………………………………………………

| PART | LESSON CONTENT | METHODOL OGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introducti on | - Recap on the previous lesson | Questions and answers | Provide answers |
| Developme nt | THE FINISHING PROCESS FINISHING - Refer to the process of article completion by enhancing the beauty and harmony of the product. ABRASIVES - Abrasives finishing materials that are used to polish materials to receive a preventive layer of chemicals such as varnish. Types of abrasives - Sand Paper ( abrasive mounted on the paper) - Emery Cloth - Abrasive mounted on the cloth - A piece of cloth coated with silicon on the back and painted with aluminium oxide. - Abrasive stone - Can mount on a machine for polishing materials. SAFETY PRECAUTIONS WHEN USING ABRASIVES - Do not enter the area where sparks are produced - Ensure there is sufficient ventilation to avoid respiratory illness - Never touch the rotating abrasive/ wheel with any part of your body - Wear safety goggles dust-proof mask, gloves and other necessary protective gear during operation - Do not use the sides of the wheel if it is designed for that purpose. SANDING WOOD WITH COATED ABRASIVES - Secure the work piece on the bench vice or in the vice - Remove any sharp edges. This helps to prevent splintering. - Sand along the grain distributing pressure evenly |  |  |
| Applicatio n | CLASS EXERCISE - Define finishing - Describe abrasives materials - State safety precautions when using abrasive | Questions And Answers | Provide answers |
| Conclusion | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CH

#### DESIGN AND TECHNOLOGY DEPARTMENT LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC: MANUFACTURING MATERIALS	SUBTOPIC: WOOD FINISHES LEARNING OUTCOMES LSBAT

- Define finishing Wood Preservative

- State the importance of Wood Preservative

- State and explain types of  Wood finishing materials

#### RATIONALE

Knowledge:	Types of finishes: Varnish, paint, stains, Steps involved when applying finishes

Skills:	Identification the correct finish for the product. Classification of finishes used on materials

Values:	Appreciation of finish on the product.  Awareness of different types of finishes.

#### TEACHING AIDS

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introd uction | - Recap on the previous lesson | Q/A | Provide answers |
| Develo pment | FINISHING WOOD PRODUCTS Wood Preservative/Wood Finishes - Refer to the application of selected materials such as stains, filler to a wooden surface. Importance of wood preservatives - Prevent the wood from absorbing; moisture, fumes and oils that may cause Shrinkage , Swelling,, Warping or discolouring - Also prevents fungal and insect attacks and in turn finishes provide a protective coating. Preparation of surface The surface should be prepared as follows: - Punch all nails below the surface of the work before planning and sanding smooth - Remove any grease and oil by sponging with lacquer thinner or suitable solvent - Sand the surface thoroughly using different grades of sandpaper ranging from rough, medium and smooth, respectively - Remove any dust from the surface. WOOD FINISHING MATERIALS Bleaches - Oxidizing agents that break down the binding substance between cell walls on the surface of wood to make it white - They are used to reduce the contrast between sapwood and heart wood - Also to remove dark stains or blemishes. - Can be applied by brushing, swabbing, or dipping the affected portion. Paints - Manufactured materials made from wood, metal, plastics or ceramics. - Oil paints are water resistant and provide greater protection against moisture entering the finished article. - Paints used for the purpose of decorating, preserving and protecting the work(wood or metal) Stain - Stain are applied prior to vanishing to give a shiny surface FOUR TYPES OF STAINS - Oil stain , Water stain , Chemical stain , Spirit stain - Varnish FOUR TYPES OF VARNISH - Oil varnish. It is high resistant to water and alcohol - Polyurethane varnish. It is very tough, hard, flexible, and resistant to chipping. And it is used on furniture, boats and floors. - Acrylic varnish. It is colourless, clear and dries in about two hours’ time. It used as a primer and a finish except on floor - Spirit varnish - Wax polish. Waxing is a rich –looking and labour intensive finish that seals and protects the wood |  |  |
| Applic ation | CLASS EXERCISE - Define finishing Wood Preservative - State the importance of Wood Preservative - State and explain four types of Wood finishing materials | Questions And Answers | Provide answers |
| Conclu sion | - Recap on the main steps and points | V/E |  |

LESSON EVALUATION

TEACHER:MWALE CH

#### DESIGN AND TECHNOLOGY DEPARTMENT LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

#### BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC: MANUFACTURING MATERIALS	SUBTOPIC: METAL FINISHIG

#### LEARNING OUTCOMES LSBAT

- Mention and explain engineering properties.

- State the importance of  metal finishing

- State Equipment used for finishing process.

#### RATIONALE

Knowledge:	Paint, polish, blueing, galvanising, Steps involved when applying finishes

Skills:	Identification the correct finish for the product. Classification of finishes used on materials

Values:	Appreciation of finish on the product.  Awareness of different types of finishes.

#### TEACHING AIDS

| PART | LESSON CONTENT | METHO DOLOG Y | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introdu ction | - Recap on the previous lesson | Q/A | Provide answers |
| Develo pment | METAL FINISHIG ENGINEERING PROPERTIES OF METAL - Mechanical properties - Concerned with the effect of applying stress to a material. - Electrical properties - Concerned with the behaviours of materials when subjected to electrical force, such as conductivity, resistivity and dielectric - Chemical properties - concerned with the changes that occur when the materials is subjected to a chemical environment such as chemical corrosion IMPORTANCE OF METAL FINISHING - Increased durability - Improved decorative appeal - Enhanced electrical conductivity - Higher electrical resistance - Higher chemical resistance EQUIPMENT USED FOR FINISHING - Acid pickle. This removes oxides and other dirt on the metal surface - Polishing powder - Wire brushes ( for removing burrs, rust and scale with strong wire scratch brushes - Softer brushes. Used for finishing the outer or inner surface of the metallic pieces - Hand buff. It give a very high quality of shine to a polished surface - Files. For removing burrs or rough spots. - Rasps. Useful for the rapid removal of wooden stock. - Buffing - The process provides a bright mechanical finish. - It is commonly applied to nonferrous metals such as aluminium, copper, brass - It can also used on plastics - Scraper - They are used to scrape or provide a smooth finish on wood (perfect on hard wood and hollow surface) Types of scraper - Hand scraper - Cabinet scraper - Curved scrapers |  |  |
| Applica tion | CLASS EXERCISE - Mention and explain engineering properties. - State the importance of metal finishing - State Equipment used for finishing process | Question s And Answers | Provide answers |
| Conclus ion | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CHIPO

KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20…. DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…  TOPIC: MANUFACTURING MATERIALS	SUBTOPIC: METAL FINISHIG LEARNING OUTCOMES LSBAT

### State and explain types of  metal finishing materials

RATIONALE

Knowledge:	Paint, polish, blueing, galvanising, Steps involved when applying finishes

Skills:	Identification the correct finish for the product. Classification of finishes used on materials

Values:	Appreciation of finish on the product.  Awareness of different types of finishes.

TEACHING AIDS

| PART | LESSON CONTENT | METHO DOLOG Y | LEARNE R ACTIVIT Y |
| --- | --- | --- | --- |
| Introd uction | - Recap on the previous lesson | Q/A | Provide answers |
| Develo pment | METAL FINISHIG METAL FINISHING MATERIALS Paints - Used for the purpose of decorating and protecting the work(metal) - Can be applied by brushing, spraying, rolling or dipping. - Colouring with heat: - Can be applied to steel - Clean the metal and slowly heat it. - Plunge the metal into the cool water when the correct colour is reached. - Hot dipping Type of coating is applied to steel by dipping the metal into the molten aluminium, zinc, tin, or lead. E.g Galvanised steel - Anodizing - This the process used to form a protective layer of aluminium oxide on aluminium parts. - Electroplating finishing - Technique whereby a thin protective metal coating is applied or deposited on a metal surface by use of an electrical current. - Galvanising - The process of applying a protective zinc coating to iron or steel, to prevent rusting. How galvanization prevent rusting - It forms a barrier that prevents corrosive substances from reaching the underlying steel or iron. - The zinc serves as a sacrificial anode so that even if the coating is scratched, the exposed steel will still be protected by the remaining zinc. - The zinc protects its base metal by corroding before iron. - The zinc surface reacts with the atmosphere to form a compact, adherent patina that is insoluble in rainwater |  |  |
| Applic ation | CLASS EXERCISE Explain ANY four types of METAL finishing materials. | Questions And Answers | Provide answers |
| Conclu sion | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CHIPO

#### DESIGN AND TECHNOLOGY DEPARTMENT LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS: 12 DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC:  WOOD LATHE MACHINE   SUBTOPIC:  WOOD LATHE MACHINE

#### LEARNING OUTCOMES LSBAT

- Define lathe machine

- Identify and explain the parts of lathe machine

#### RATIONALE

Knowledge:	headstock, tailstock, tool rest, headstock spindle, safety precaution, turning tools and maintenance

Skills:	Interpretation of operations and processes to be followed on the lathe machine Values:	Awareness  of different types of operations performed on the lathe machine TEACHING AIDS

| PART | LESSON CONTENT | METHODOL OGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introdu ction | - Recap on the previous lesson | Q/A | Provide answers |
| Development | LATHE MACHINE The term used to describe the machinery used to decorate wood or metal article PARTS OF THE CENTRE MACHINE - THE HEADSTOCK - This is the live end of the machine because motion is initiated there - It drives the work piece throughout the constant rotation of the part by means of a v belt in the pulley - HEADSTOCK SPINDLE This is usually threaded on both ends. The thread on the inside or bed is right hand thread while the thread outside on the face plate is left hand. This makes the chucks self- tightening because the threads are rotating in opposite directions - THE TAILSTOCK - This part is used to support work from the headstock. It is movable along the bed to the required dimensions. - It usually carries a tail stock centre - THE BED Consist of a pair of accurately machined slide ways. The tail stock and the tool rest slide along this component - TOOL REST - Are used to support the turning tools in use. They can be adjusted in any required position along the bed and at the same time it can be raised or lowered - FRAME This is the whole body of the machine. - LATHE CENTRES Types of centres a. Cone centre , Fork centre , Spanner flat ring centre , Screw centre - CHUCKS - Are used for holding timber while turning - Are useful when working with a single centre rather than between centres, they provide a strong grip Types of chucks - jaws- are useful for holding round piece of timber - 4-jaws can grip a bulky object for turning |  |  |
| Applicatio n | CLASS EXERCISE - Draw and label the parts of the lathe ( wood) - State two parts of the lathe which slide along the bed - State two types of jaws chuck | Questions And Answers | Provide answers |
| Conclus ion | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CHIPO

## DESIGN AND TECHNOLOGY DEPARTMENT

#### LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS: 12 DATE:……/……./20….	DURATION:………….

#### BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC:  WOOD LATHE MACHINE	SUBTOPIC:  LATHE OPERATION

#### LEARNING OUTCOMES LSBAT

- Explain the three major lathe operation

- Explain types of  turning tools

- State the Safety Precaution when are you using lathe machine and maintenance of the lathe machine

#### RATIONALE

Knowledge:	Precaution, turning tools and maintenance

### Skills:	Identification of turning tools Classification of tools to be used

Values:	Awareness of different types of operations performed on the lathe machine

#### TEACHING AIDS

……………………………………………………………………………………………………………………………………………

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introdu ction | - Recap on the previous lesson | Questions and answers | Provide answers |
| Development | LATHE MACHINE TURNING TOOLS - Scraping tools are used to finish a turned surface to a particular shape - Parting tools are used to cut through completed work on the lathe in order to remove the waste from either end - Skew chisel is used to shape and finish work. It makes very clean, smooth cuts. - Gouges are used to take the bulk off the wood THREE MAJOR LATHE OPERATION - Turning between centres Describe the process whereby the wood is mounted in between headstock and tailstock of the lathe machine. - Face plate or bowl turning The process of producing circular/ spherical shapes, edges and surfaces, best for bowls and dishes. It also used to produce flat but circular articles such as lamps stand bases and other disc shaped - Long boring Boring is the process of machining a cylindrical shape inside the previously drilled hole. Take light cuts only It is done when a very accurate hole diameter is needed MAINTENANCE - Oil the machine every time you state work - Sharpen the turning tools frequently, - Always keep machine component tightened up, - Mount the machine firmly on the ground in the workroom SAFETY PRECAUTION - Keep working surface clean of scraps, tools and materials. - Keep floor around lathe clean and free of oil and grease. - Keep all lathe cutting tools sharp - Make sure the lathe has a start/stop button within easy reach of the operator - Guard all power transmission parts. - Don’t use the lathe if you have notice any problem - Always use safety goggles to protect your eyes from spit out chips and not hot particles in case of metal turning - Ovoid overfeeding the machine - Protect your body with the correct safety attire |  |  |
| Applicat ion | CLASS EXERCISE - Explain the three major operation on the lathe machine - State 4 safety precaution when using lathe machine - Explain the maintenance of the lathe machine | Questions And Answers | Provide answers |
| Conclu sion | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CHIPO

#### DESIGN AND TECHNOLOGY DEPARTMENT LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS: 12 DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC: CENTRE LATHE MACHINE  SUBTOPIC:  CENTRE LATHE MACHINE LEARNING OUTCOMES LSBAT

- Define lathe machine

- Identify and explain the parts of lathe machine

#### RATIONALE

Knowledge:	headstock, tailstock, carriage, bed,

Skills:		Interpretation of operations and processes to be followed on the lathe machine Values:	Awareness  of different types of operations performed on the lathe machine TEACHING AIDS

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introdu ction | - Recap on the previous lesson | Q/A | Provide answers |
| Development | CENTRE LATHE MACHINE A lathe machine is used for cutting, drilling, knurling and other related machine PARTS OF THE CENTRE MACHINE - THE HEADSTOCK - This is the live end of the machine because motion is initiated there - It is serve as holding device for the gear chain, spindle, driving pulley etc. It is also made by cast iron. - THE TAILSTOCK - It is mounted on the bed of the lathe and slide along the bed - The main function is to provide bearing and support to the job which is being worked between - The tailstock unit consists of Dead centre, barrel clamp, tail stock clamp, hand wheel - THE BED - Provide the foundation for the whole machine and holds the headstock, tailstock, carriage in alignment. - THE CARRIAGE - This form the base of unit which supports the cutting tool - It provide two types of movement manually and by power feed and it slide over the bed ways - It provide cross feed of the tool and its movement is perpendicular to the centre of the machine - Compound Slide or rest, Compound rest is set up over the cross slide and it can move in a circular path. - Tool Post: Its function is to hold the tool or the tool holder - Cross Slide: Its function is to provide cross feed of the tool and its movement is perpendicular to the center of the lathe machine. OTHER PARTS - LEAD SCREW= It is used to move the carriage automatically during thread cutting 14. CHUCK = used to hold the work piece - APRON - It consist all controlling and moving mechanism of carriage. Contains the feed mechanism and control for the automatic movement of the cross slide. |  |  |
| Applicat ion | CLASS EXERCISE - Identify and explain five main parts of the centre lathe ( metal) - State two parts of the lathe which slide along the bed | Questions And Answers | Provide answers |
| Conclus ion | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CHIPO

## KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

#### LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS: 12 DATE:……/……./20….	DURATION:………….

#### BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC:  CENTRE LATHE MACHINE	SUBTOPIC:  CENTRE LATHE OPERATION MACHINE

#### LEARNING OUTCOMES LSBAT

- Explain the holding work on the centre lathe

- Explain centre lathe process

#### RATIONALE

Knowledge:	Facing Off , Plain Turning , Tapering,  Drilling,  Boring  And Knurling

Skills:		Interpretation of operations and processes to be followed on the lathe machine Values:	Awareness of different types of operations performed on the lathe machine TEACHING AIDS

……………………………………………………………………………………………………………………………………………

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introdu ction | - Recap on the previous lesson | Q/A | Provide answers |
| Development | CENTRE LATHE MACHINE HOLDING WORK ON THE LATHE - 3-JAW CHUCK (SELF-CANTERING) All jaws opening and closing together as the scroll is turned with the key - 4 JAW CHUCK INDEPENDENT Its operate individually. This permit irregular shaped work (square, rectangular, octagonal) to be centred CENTRE LATHE MACHINE PROCESS FACING OFF End or face of the stock is machined square in order to produce flat, smooth surface. It should be done at high speed PLAIN TURNING It is done when the work must be reduced in diameter. Lubricate the tailstock (dead centre) in order to ovoid over heating which can result in burn TAPERING When the of a piece changes uniformly from one end to the other. DRILLING Making holes in work piece BORING Boring is the process of machining a cylindrical shape inside the previously drilled hole. Take light cuts only It is done when a very accurate hole diameter is needed KNURLING To provide a gripping surface Knurling is the process of impressing a straight or diamond shapes pattern onto the surface of a work piece such as a centre punch to give a better grip. USES OF KNURLING Allow hands or finger to get a better grip on the knurled object Also adds a decorative pattern to the material Knurling process THREAD CUTTINGS Thread can be cut on the lathe machine FILLING AND POLISHING - Filling and polishing of work piece can be done on lathe machine |  |  |
| Applicati on | CLASS EXERCISE - Explain the following process Facing off , Plain turning , Boring , Knurling - What is the difference between 3- jaw chuck and 4-jaw chuck | Questions And Answers | Provide answers |
| Conclu sion | - Recap on the main steps and points | V/E |  |