- Explain hydraulics

# Explain the operation of hydraulics

- Describe the basic principles and cycles of hydraulic systems

## RATIONALE

Knowledge:	Principles of hydraulic systems: hydraulic transmission of forces (Syringes, fluids, tints, elastic tubes, clips) Skills:	Identification of basic operations of hydraulic systems. Application of hydraulic principles when designing Values:	Awareness of principles of hydraulic systems. Appreciation of hydraulics in technology

## TEACHING AIDS

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introduct ion | - Recap on the previous lesson | Q/A | Provide answers |
| Development | HYDRAULICS - These are system that uses oil, or fluids instead of air to transmit motion and force. - Refers to power produced by moving liquids. - Modern hydraulics is the use of confined liquid to transmit power, multiply force, or produce motion. OPERATION OF HYDRAULICS SYSTEM - When force is applied to piston it will exert pressure to the oil in the cylinder which then the oil is forced to move along the pipe or tube. The pressure created is measure of force over a given area BASIC PRINCIPLES AND CYCLES OF HYDRAULIC SYSTEMS - Pascal’s Law: “Pressure applied on a confined fluid is transmitted in all directions with equal force on equal areas”. Examples - A closed toothpaste tube can be considered as an example for a simple hydraulic system. - The toothpaste can be considered as a hydraulic fluid working inside the system which is confined. - Four or five holes are made on the tube. - By keeping the cap of the toothpaste tube closed, apply pressure at a particular point on the tube. This makes the toothpaste to come out from all the holes evenly. - This is a simple example to understand Pascal’s law. |  |  |
| Applicati on | CLASS EXERCISE - Explain hydraulics - Explain the operation of hydraulics - Describe the basic principles and cycles of hydraulic systems | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |

LESSON EVALUATION

- Explain the Basic Components of a Hydraulic System

- state application of hydraulics

- Outline the Advantages and Disadvantages of Hydraulic Systems?

RATIONALE

Knowledge:	Advantages and disadvantages of hydraulics

Skills:	Identification of basic operations of hydraulic systems. Application of hydraulic principles when designing

Values:	Awareness of principles of hydraulic systems. Appreciation of hydraulics in technology

## TEACHING AIDS

| PART | LESSON CONTENT |  |  |  | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- | --- | --- | --- |
| Introduct ion | - Recap on the previous lesson |  |  |  | Q/A | Provide answers |
| Development | HYDRAULICS BASIC COMPONENTS OF A HYDRAULIC SYSTEM - Fluid: oil for hydraulic systems, air for pneumatics. - Reservoir: storage tank. - Hydraulic pump (compressor in pneumatics): converts the mechanical energy into hydraulic energy by forcing fluid from the reservoir into the system. - Fluid lines: transport the fluid to and from the pump through the hydraulic system. - Valves: control pressure, direction and flow rate of the hydraulic fluid. - Actuator: converts hydraulic energy into mechanical energy to do work. APPLICATION OF HYDRAULICS - Automobile garage, Petrol pumps, Measuring weights of heavy-lift trucks, Hydraulic cranes, Automobile steering gears Automobile brake, (disc brakes) lifts Hydraulic Jack ADVANTAGES AND DISADVANTAGES OF A HYDRAULIC SYSTEM |  |  |  |  |  |
|  |  | Advantages | Disadvantages |  |  |  |
|  |  | - Uses small components to transfer large forces with consistent power output. - Produces even and smooth movements under varying loads since the fluids aren't compressible and flow rates can be accurately controlled with valves. - Is easy to control and regulate with pressure, directional and flow control valves. - Performs well in hot environments. | - Pumps, valves, transmission networks and actuators are expensive. - They can pollute the workplace with leaks, which may cause accidents or fires. - They're not suitable for cycling at high speeds. - Hydraulic fluids are sensitive to dirt contamination and must be tested regularly. - Ruptures of high-pressure lines can cause injuries |  |  |  |
| Applicati on | CLASS EXERCISE - state application of hydraulics - Outline any two Advantages and Disadvantages of Hydraulic Systems? |  |  |  | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points |  |  |  | V/E |  |

LESSON EVALUATION

- Define pneumatics

- Describe the basic principles and cycles of Pneumatic systems

- Explain the Components of a Pneumatic System

RATIONALE

Knowledge:	Principles of pneumatic systems: pneumatic circuits (bicycle pumps)

Skills:	Application of operations and principles when designing pneumatic circuits Interpretation of pneumatic principles and operations

Values:	Creative thinking in designing pneumatic circuits. Appreciation of the importance of pneumatics in technology Awareness of principles and operations of pneumatics

## TEACHING AIDS

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introduct ion | - Recap on the previous lesson | Q/A | Provide answers |
| Development | PNEUMATICS - pneumatics are control systems which use compressed air in order to do the work - pneumatic is applied in Doors of buses and trains, air tanks of trucks, large vehicles brakes BASIC PRINCIPLES AND CYCLES OF PNEUMATIC SYSTEMS - Pneuma’ means air. Pneumatics is all about using compressed air to do the work. - Compressed air is the air from the atmosphere which is reduced in volume by compression thus increasing its pressure. - Compressed air is mainly used to do work by acting on a piston or vane. COMPONENTS OF A PNEUMATIC SYSTEM - Air compressor - The air compressor draws air from the atmosphere, pressurizes it and stores the compressed air in a tank for release to the transmission system. - Prime driver - A prime driver, such as an electric motor or a gas-powered engine, provides the power to an air compressor. - Control devices - Valves regulate the pressure and control flow and direction. - Air tank - A tank holds compressed air for delivery to mechanical devices. - Actuators - These are devices that take the energy from compressed air and convert it into mechanical movements. - Transmission system - A network of pipes and tubing transports the compressed air to actuators. |  |  |
| Applicati on | CLASS EXERCISE - What is pneumatics - Where is pneumatic is applied - Describe the basic principles and cycles of Pneumatic systems | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |

LESSON EVALUATION

## LEARNING OUTCOMES LSBAT

- Outline the Advantages and Disadvantages of Pneumatic Systems?

RATIONALE

Knowledge:	Advantages and disadvantages of Pneumatic Systems.

Skills:	Application of operations and principles when designing pneumatic circuits Interpretation of pneumatic principles and operations

Values:	Creative thinking in designing pneumatic circuits. Appreciation of the importance of pneumatics in technology Awareness of principles and operations of pneumatics

## TEACHING AIDS

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introduct ion | - Recap on the previous lesson | Q/A | Provide answers |
| Development | PNEUMATIC ADVANTAGES OF PNEUMATIC SYSTEMS - Efficiency - The supply of air is free and unlimited. Compressed air is easy to store, transport and can be released to the environment without costly treatments. - The ability to operate at higher speeds - Cleanliness - Pneumatic systems are preferred in workplaces that need high levels of cleanliness. - Less Costly - Maintenance costs are lower compared to hydraulic systems. - Safer to operate - Pneumatic systems are safe to use in inflammable environments without dangers from fire or explosions. - Pneumatic components don't overheat or catch on fire when overloaded. - Able to function in harsh environments - Dust, high temperatures and corrosive environments have less effect on pneumatic systems compared to hydraulics. DISADVANTAGES OF PNEUMATIC SYSTEMS - Noisy - Air compressors generate more noise, - Rough movement - Because air is compressible, the motion of pneumatic actuators can be rough, which reduces the accuracy of the movements of the system. - Need pre-treatment of air - Before use, air needs processing to remove water and dust particles. If this isn't done, the increased friction between the control devices and moving components will wear out the part and require premature repair or replacement. |  |  |
| Applicati on | CLASS EXERCISE Outline any three Advantages and Disadvantages of Pneumatic Systems | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |

LESSON EVALUATION