TEACHER:MWALE CH.

# DESIGN AND TECHNOLOGY DEPARTMENT

### LESSON PLAN

| Cables to be used | Cable size | Recommended fuse |
| --- | --- | --- |
| General lighting circuit | 1.0mm/1.5mm twin earth | 5/6 amp |
| general ring main power | 2.5 mm twin + earth | 30/32 amp |
| General radial main power circuit | 2.5 mm twin + earth | 15/16 amp |
| Cookers/geyser up to 40 amp maxi load | 6 mm twin +earth | 30/40 amp |

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

### BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC:  INSTALLATION AND APPLIANCE	SUBTOPIC:  DOMESTIC INSTALLATION

### LEARNING OUTCOMES LSBAT

- Define wiring

- Identify the recommended cables used in domestic installation

- Explain function of live, neutral, earth wires and fuse

### RATIONALE

Knowledge:	Recommended cables, radial main power, ring main power,

Skills:	Identification, design, drawing interpretation

Values:	Problem solving, Team work

### TEACHING AIDS

- Socket outlet, Cables, Circuit Breaker, and Chalk Board

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Intro ducti on | - Recap on the previous lesson | Questions and answers | Provide answers |
| Development | DOMESTIC INSTALLATION WIRING A BUILDIND Refer to installing suitable circuits which will be used to supply electricity to different appliance to be used in the building SAFETY PRECAUTION - Always use the correct size fuse - Always use the correct size cable SIZE OF DOMESTIC INSTALLATION CABLES WIRES - LIVE WIRE carries electric current to the appliance - NEUTRAL WIRE complete the circuit by forming a path for the current back to the main - EARTH WIRE - To protect building, machinery and appliances under fault conditions ii.To safe human life from danger of electricity shock or death by blowing a fuse - FUSE To prevent short circuit that can cause fire and damage to electrical appliance | Questions And Answers Demonstra tion V/E | Taking note of important points Provide answers |
| Application | CLASS EXERCISE - Explain the function of the following - Fuse , Earth wire , Live wire and Neutral wire - Identify the recommended cable size and their amps for the following domestic installation - General lighting circuit - general ring main power - Cookers/geyser up | Questions And Answers | Provide answers |
| Concl usion | - Recap on the main steps and points | V/E |  |

### TEACHE:

DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

### BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC:  INSTALLATION AND APPLIANCE	SUBTOPIC:  INSTALLING SOCKET OUTLETS

### LEARNING OUTCOMES LSBAT

- State and explain the installing socket outlets

- State advantages and disadvantages of installing socket outlets

### RATIONALE

Knowledge:	Radial main power, ring main power,

Skills:	Identification, design, drawing interpretation

Values:	Problem solving, Team work

### TEACHING AIDS

Socket outlet, Cables, Circuit Breaker, and Chalk Board

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Intro ducti on | - Recap on the previous lesson | Questions and answers | Provide answers |
| Development | INSTALLING SOCKET OUTLETS RING MAIN POWER CIRCUIT - Cables runs from the consumer unit to the first outlet to the next, from there continuous to the next and so on until the last outlet - Cables continue back to the consumer where it connect to the same terminal - Protect by 30A MCB circuit breaker - Maximum area is 100 m2 - Cover the length of 60m - 30A for cartridge fuse and 32A for MCB fuse ADVANTAGE OF RING MAIN POWER CIRCUIT - Current can reach a socket by two routes and each wire has to carry only half the current DISADVANTAGE RING MAIN POWER CIRCUIT - More cables are needed to join the sockets in the ring - It is expensive RADIAL MAIN POWER CIRCUIT - Commences at the distribution board circuit hopes in various socket outlets and ends at the last socket - 2.5mm cables and rating of MCB uses 15 A - The height of the sockets outlet is usually about 30 cm to about 50 cm from the floor ADVANTAGE OF RADIAL MAIN POWER - The initial cost is low - Simplest as fed at only one end - Require less amount of cables - It has low maintenance DISADVANTAGE OF RADIAL MAIN POWER - The of distributor near to the substation gets heavily loaded | Questions And Answers Demonstra tion V/E | Taking note of important points Provide answers |
| Applicatio n | CLASS EXERCISE - State two advantages and disadvantages of the following - Ring main power circuit - Radial main power circuit | Questions And Answers | Provide answers |
| Concl usion | - Recap on the main steps and points | V/E |  |

NAME:MWALE CHIPO

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC: DOMESTIC INSTALLATION	SUBTOPIC:  INSTALLING LIGHTING CIRCUIT LEARNING OUTCOMES LSBAT

- State two types of connection

- Explain the characteristic of series and parallel connection

- Identify and explain three types of lighting circuit (switches)

### RATIONALE

Knowledge:   One Way Lighting, 2 Way Lighting Circuit, Intermediate Lighting Circuit,

Skills:	Identification, Design, Drawing Interpretation

Values:	Problem solving, Team work

### TEACHING AIDS

……………………………………………………………………………………………………………………………………………

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introdu ction | - Recap on the previous lesson | Questions and answers | Provide answers |
| Development | TYPES OF LIGHTING - ONE WAY LIGHTING CIRCUIT: If a light switch is ‘1 way’ this means it is just a on/off switch. This is the simplest type of switch that you can get. The single switch controls a single light (or lighting circuit). - TWO WAY LIGHTING CIRCUIT: A ‘2 way’ switch means there is another switch controlling the same light. These are often used on a stair case, large room with switches by each door. - THREE INTERMEDIATE LIGHTING CIRCUIT: If you need to control the lights from three places, say you have 3 entrances to a large room and need a light switch next to each one, you will need an intermediate switch. The light(s) can be switched on and off from anyone of 3 places. | Questions And Answers Demonstra tion V/E | Taking note of important points Provide answers |
| Applicati on | CLASS EXERCISE - Identify and explain three types of lighting circuits (switches) - Why are 2 bulbs dimmer in a series circuit? | Questions And Answers | Provide answers |
| Conclu sion | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CH.

### DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:…

### BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC: DOMESTIC INSTALLATION	SUBTOPIC:  INSTALLING LIGHTING CIRCUIT

### LEARNING OUTCOMES LSBAT

- State two types of connection

- Explain the characteristic of series and parallel connection

### RATIONALE

Knowledge:   Series, Parallel, One Way Lighting, 2 Way Lighting Circuit, Intermediate Lighting Circuit,

Skills:	Identification, Design, Drawing Interpretation

Values:	Problem solving, Team work

### TEACHING AIDS

| PART | LESSON CONTENT | METHO DOLOG Y | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introdu ction | - Recap on the previous lesson | Q/A | Provide answers |
| Development | INSTALLING LIGHTING CIRCUITS TYPES OF CONNECTION - SERIES CONNECTION - Is circuit in which several resistance eg lamps are connected one after the other. Such connection is also called end to end - 1 path for current CHARACTERISTICS OF SERIES CIRCUITS: - current is the same everywhere (ammeter operation) IT = I1 = I2 =I3 - voltage is different across different resistances VT = V1 + V2 + V3 (voltmeter operation) - total resistance is equal to the sum of the resistances R = R1 + R2 + R3 - A break in the circuit stops all current OBSERVATION - When the switch is closed, dim lights will appear in all three lamps and 40W lamps will be brighter than other two because it has the lowest resistance. - Bulbs dimmer in a series circuit because the current going through them is smaller because two bulbs in series have a higher resistance than a single bulb PARALLEL CONNECTION - More than 1 path for current CHARACTERISTICS OF PARALLEL CIRCUITS: - Current is different in different resistances, but the total current is equal to the sum of the currents in all the branches. Eg all lamps experienced same voltage IT = I1 + I2 + I3 - voltage is the same across all resistances VT = V1 = V2 =V3 - total resistance is less than the sum of the individual resistances VT/RT = V1/R1 +V2/R2 + V3/R3 AND 1/RT = 1/R1 + 1/R2 + 1/R3 - a break in the circuit only stops current in the branch where the break occurs | Questions And Answers Demonstra tion V/E | Taking note of important points Provide answers |
| Applicat ion | CLASS EXERCISE - Explain the characteristic of the following - Series connection - Parallel connection | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |