TEACHER: MWALE CHIPO

##### KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

##### BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC: IMPELLING TOOLS	SUBTOPIC:  IMPELLING TOOLS (HAMMERS)

##### LEARNING OUTCOMES LSBAT

- Define impelling

- Identify different types of impelling tools.

- State and explain different types of hammers

- Explain precaution when you are using hammer

##### RATIONALE

Knowledge:	Types and uses:  hammers,   precaution when you are using hammer

Skills:	Identification,

Values:	Problem solving, Appreciation and Application.

##### TEACHING AIDS

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introdu ction | - Recap on the previous lesson | Q/A | Provide answers |
|  | IMPELLING TOOLS Impelling tools are tools that are used to fixing or driving pieces of work temporarily or permanently. EXAMPLES OF IMPELLING TOOLS - Hammers - Mallet - Screw driver - Spanner - Pincer HAMMER - Hammer heads are made of high carbon, heat-treated steel for strength and durability TYPES OF HAMMERS Claw hammer - It is used for pounding nails into, extracting nails - The handle is made of wood or steel and a hand grip of rubber or leather Parts of claw hammer - Claw. It is used for removing nails - Face. Used to drive nails - Handle. For holding when you are using it. Warrington or cross-pane hammer - Has a wedge at one end of its head - It is used for forging and riveting Ball-pein hammer - It is designed for metal work - It is used to drive punches and cold chisels, set rivets and to shape metal. - Its head is hardened and Parts of ball pane hammer Club or lump hammer - it has a double faced head, and is useful for light demolition work - Used for the heavier jobs, such as driving in stakes or to break up concrete, stone or masonry. Precaution when using hammers - Hammers heads should never be struck against one another or any hardened metal surface. - Ensure that the head of the hammer is firmly attached to the handle. - Replace loose, cracked or splintered handles. - Keep the work area clear of debris. - Do not use a hammer with a loose or damaged handle. |  |  |
| Applicati on | CLASS EXERCISE - Define impelling - Identify different types of impelling tools. - State any three safety precautions when you are using hammer | Questions And Answers | Provide answers |
| Conclus ion | - Recap on the main steps and points | V/E |  |

TEACHER: MWALE CHIPO

### KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

##### LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

##### BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC: IMPELLING TOOLS	SUBTOPIC:  MALLET, SCREWDRIVER, SPANNER AND PINCER

##### LEARNING OUTCOMES LSBAT

- Identify and explain different types of impelling tools.

##### RATIONALE

Knowledge:	Types and Uses:  Screw Drivers, Mallets, Spanner, Pincer

Skills:	Identification,

Values:	Problem solving, Appreciation and Application.

##### TEACHING AIDS.

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introd uction | - Recap on the previous lesson | Questions and answers | Provide answers |
| Develo pment | IMPELLING TOOLS MALLET - A mallet has a large head, often made of rubber or wood Types of mallet a. Carpenter’s mallet - It is used to strike a chisel or wedge. - It is also used to tap wood joints together where a metal-faced hammer would cause damage or bruising b. Rubber mallet - It used when a softer blow is required - They are used to bend and shape sheet metal, since they do not leave marks - They are also used for forcing tight-fitting wooden parts together SCREW DRIVER - It is used for turning screws (inserting and withdrawing screws) - The shaft made of tough steel to resist bending or twisting - The tip may be hardened to resist wear - Handle made of wood, metal, or plastics and may be hexagonal, square or oval in cross section to improve the grip and prevent the tool from rolling when set down Types of screw drivers - London pattern screwdriver. - Has a flat blade and flat faces on the handle - Cabinet pattern screw driver. - Has rounded steel blade and wooden handle - It has a thinner shank than most screw drivers. The thin shanks allows the tip to reach screws in deep holes without damaging the surrounding wood PINCER - Used in many situations where a mechanical advantage is required to pinch cut or pull an object. - Parts of a pincer (Jaws, pivot, handle, cutting edges ) SPANNER |  |  |
| Application | CLASS EXERCISE - Draw and label the parts of the following tools - Mallet - Screwdriver - Explain the use of pincer. - State two types of screwdrivers. | Questions And Answers | Provide answers |
| Conclu sion | - Recap on the main steps and points | V/E |  |

TEACHER: MWALE CHIPO.

### KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC:	HEAT TREATMENTS	SUBTOPIC:  HEAT TREATMENTS OF STEEL LEARNING OUTCOMES LSBAT

#### Define  Heat Treatments

- Explain  the objectives heat treatment

- Explain hardening  heat treatment

RATIONALE

Knowledge:	Heat treatment (Hardening)

Skills:	Identification,

#### Values:	Problem solving, Appreciation and Application:

TEACHING AIDS

| PART | LESSON CONTENT | METHODOL OGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introducti on | - Recap on the previous lesson | Questions and answers | Provide answers |
| Develop ment | HEAT TREATMENTS OF STEEL - Heat treatment is a heating and cooling process of a metal or an alloy in the solid state with the purpose of changing their properties. THREE PHASES OF HEAT TREATMENTS - Heating of the metal, - Soaking of the metal - Cooling of the metal. NOTE. - The rate of cooling is the major controlling factor. E.g. Rapid cooling the metal results in hard structure OBJECTIVES HEAT TREATMENT - It improves mechanical properties such as ductility, strength, hardness, toughness. - It increases wear resistance, - It removes gases. - It improves electrical and magnetic properties. - It improves weldability. METHODS OF HEAT TREATMENT OF METALS HARDENING - The process used to increase the hardness of a metal. - Its main purpose is to resist wear and increase the cutting ability PROCEDURE - Heating of the metal up to hardening temperature (over 723oC) depending on the carbon content. - Holding the temperature according to the grade and size of steel - Sudden cooling down (quenching) of the metal NOTE - Quenched materials is water, oil or molten salt bath USE OF HARDENING METAL - Construction materials - Machine cutting tools .e.g. drill bits. Taps and lathe tools - Knife blades- high hardness blade keeps a sharp edge - Bearings |  |  |
| Applicati on | CLASS EXERCISE - Define heat treatment - State any two objectives heat treatment - Explain the procedure of hardening metal | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |

2

TEACHER: MWALE CHIPO

# KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY CLASS:… DATE:……/……./20….	DURATION:……

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC:	HEAT TREATMENTS	SUBTOPIC:  HEAT TREATMENTS OF STEEL LEARNING OUTCOMES LSBAT

## Explain Annealing and Normalising Heat Treatment

RATIONALE

Knowledge:	Heat treatment (Annealing, Normalising,)

Skills:	Identification,

Values:	Problem solving, Appreciation and Application: TEACHING AIDS

| PART | LESSON CONTENT | METHO DOLOG Y | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introduc tion | - Recap on the previous lesson | Question s and answers | Provide answers |
| Development | HEAT TREATMENTS OF STEEL ANNEALING - Heating treatment that alters the structure of material causing changes in properties such as strength, hardness and ductility. - To reduce its hardness/make it softer. OBJECTIVES OF ANNEALING - Soften the steel. /relieve internal stresses. - Refine grain size./ Improve machinability - It increases ductility of steel - It changes electrical and magnetic properties of steel - Remove internal tensions. PROCEDURE OF ANNEALING - Consist of heating to temperature (8000C and 9500C) depending on the type of steel - Followed by slow cooling NORMALISING OBJECTIVES OF NORMALISING - To soften metals - Relieving internal stresses , - Refine grain size - Improving the mechanical properties - Improve structure of weld - Prepare steel for sub heat treatment - To leave the material in normal state. PROCEDURE OF NORMALISING - Steel is heated to (8000C and 9000C) - Cooled it in still air. |  |  |
| Applicat ion | CLASS EXERCISE - State any three objectives of following annealing and normalising - Explain the procedure of annealing and normalising heat treatment | Question s And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |

3

TEACHER: MWALE CHIPO

### KENNETH KAUNDA  SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:… DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC:	HEAT TREATMENTS	SUBTOPIC:  HEAT TREATMENTS OF STEEL LEARNING OUTCOMES LSBAT

## Explain the following heat treatment

- Case Hardening

- Tempering

### RATIONALE

Knowledge:	Heat treatment (Tempering, Casehardening)

Skills:	Identification,

#### Values:	Problem solving, Appreciation and Application:

TEACHING AIDS

| PART | LESSON CONTENT | METHO DOLOG Y | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introduc tion | - Recap on the previous lesson | Question s and answers | Provide answers |
| Development | HEAT TREATMENTS OF STEEL TEMPERING - Tempering is the process which is used to increase the toughness of iron-based alloys. - It is performed after hardening, to reduce some of the excess hardness, brittleness and unwanted induced stress. PROCEDURE OF TEMPERING - Steel is heated to 150-500°C - Cooled it in still air. METHOD OF QUENCHING - Immersing the hot steel in water, oil, or forced-air. CASE HARDENING - The process of hardening the surface of a metal object while allowing the metal deeper underneath to remain soft. - This form a layer of harder metal called the “case”, at the surface OBJECTIVES OF CASE HARDENING - To have a hard surface and tough core. - Get outer surface hard where inner core is kept tough USES OF CASE HARDENING METAL - Used for production of parts required to be hard and impact – resistant on the surface and softer and denser in core. - E.g. Gears, shafts, piston pins, chains gears and rollers, roller bearing and cutting tools. |  |  |
| Applicat ion | CLASS EXERCISE Explain the following heat treatment - Case Hardening - Tempering | Question s And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |

LESSON EVALUATION