TEACHER:MWALE CH

## KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:………. DATE:……/……./20….  DURATION:……

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC:	ELECTRICAL PRINCIPLES	SUBTOPIC: TRANSFORMER LEARNING OUTCOMES LSBAT

- Define Transformer

- State the purpose of a Transformer

- Draw and explain the Main Constructional Parts of Transformer

- Explain the Principle on mutual induction

## RATIONALE

Knowledge:	Main constructional parts of transformer

Skills:	Identification of physical features of a transformer Values:	Awareness of construction of transformers TEACHING AIDS

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introduct ion | - Recap on the previous lesson | Q/A | Provide answers |
|  | TRANSFORMER - Transformer is a static device which transfers electrical energy (voltage or current) from one circuit to another. - Transformer is a device that makes use of mutual induction to vary voltage. PURPOSE OF A TRANSFORMER - The purpose of a transformer is to change electrical voltage to a different value. MAIN CONSTRUCTIONAL PARTS OF TRANSFORMER The essential parts in a transformer are Magnetic Core of Transformer - The transformer core is made of sheet steel or silicon steel with high permeability and low hysteresis - The sheets are laminated and coated with an oxide layer to reduce eddy current loss Two windings - Primary windings of transformer, - which receives electrical energy - it produces magnetic flux when it is connected to electrical source) - Secondary windings of transformer - It delivers electrical energy and gives the desired output of the Transformer. - They are made of copper - The windings are stationary and the magnetic flux is changing Notice - That the two coil windings are not electrically connected but are only linked magnetically PRINCIPLE OF OPERATION - The transformer is working under the principle of electromagnetic induction |  |  |
| Applicati on | CLASS EXERCISE - Define Transformer - State the purpose of a Transformer - Draw and explain the Main Constructional Parts of Transformer | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CH

## KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC:	TRANSFORMER	SUBTOPIC: TRANSFORMER LEARNING OUTCOMES LSBAT

- Identify types of Transformer core

- Identify and explain types of Transformers,

- Explain maintenance of transformers

## RATIONALE

Knowledge:	Types of transformer cores, Types of transformers, Single phase and 3 phases, Step up and step down.

Skills:	Identification of physical features of a transformer

Values:	Awareness of different types of transformers and transformer cores

## TEACHING AIDS

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introduct ion | - Recap on the previous lesson | Q/A | Provide answers |
| Develop ment | TRANSFORMER TYPES OF TRANSFORMER CORES - According to the Transformer construction, they are classified into two types. CLOSED-CORE TYPE TRANSFORMER. - In the core type Transformers, the windings surround the core SHELL-CORE TYPE TRANSFORMER - In shell type Transformers, the core surrounds the windings. CORE-TYPE SHELL-CORE TYPES OF TRANSFORMER SINGER PHASE TRANSFORMER - A single-phase Transformer can operate to either increase or decrease the voltage applied to the primary winding Step-Up Transformer - Used to “increase” the voltage on its secondary winding with respect to the primary - More number of turns in secondary winding and fewer number of turns in primary winding Step-Down Transformer. - It is used to “decrease” the voltage on the secondary winding with respect to the primary - More number of turns in primary and fewer number of turns in secondary Impedance Transformer - When Transformer produces the same voltage on its secondary as is applied to its primary winding. THREE PHASE TRANSFORMER - Three-phase, also written as 3-phase or 3φ supplies are used for electrical power generation, transmission, and distribution, as well as for all industrial uses. MAINTENANCE OF TRANSFORMERS - Inspections include visual checks for cleanliness, cracked insulators and bushings, oil condition, and condition of grounds - It is also important to keep the area around them clear. - Transformer rooms and vaults should be ventilated. |  |  |
| Applicati on | CLASS EXERCISE - Explain the following transformers - Step up transformer and Step down transformer - Explain the maintenance of transformers | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CH

# KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:……

# BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC:	TRANSFORMER	SUBTOPIC:	TRANSFORMATION RATIO, EMF EQUATION

# LEARNING OUTCOMES LSBAT

- Find the ratio of  the given Transformer.

RATIONALE

Knowledge:	Basic calculations: Transformation ratio, Emf equation

Skills:	Calculation of transformation ratios

Values:	awareness of calculation or transformer ratio

TEACHING AIDS

| PART | LESSON CONTENT | METHODOL OGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introduc tion | - Recap on the previous lesson | Q/A | Provide answers |
| Develop ment | TRANSFORMATION RATIO, EMF EQUATION. A TRANSFORMERS TURNS RATIO - NP/NS=VP/VS= n = Turns Ratio  𝑁𝑃 = 𝑉𝑃 = 𝐼𝑆 𝑁𝑆 𝑉𝑆 𝐼𝑃 HOW TO FIND TRANSFORMER RATIO EXAMPLE. 1 The transformer has 16v and 8 Turns on its primary winding, 8v and 2 Turns on its secondary winding. Calculate the transformer ratio. Data - NP = 8TURNS - VP= 16V - NS= 2TURNS - VS= 8V Therefore NP/NS or VP/VS - TRANSFORMER RATIO 8/4 = 2:1 ANSWER - We call this type of transformer as step down transformer because we are stepping down the voltage by two to one ratio. EXAMPLE. 2 A battery charger for a 5volts mobile phone battery has 4000 Turns on its primary winding (coil) and 100 turns on its secondary winding. What is the main supply voltage? a. 100v - 240 v b. 50v - 80v c. 12v - 20 SOLUTION Data NP = 4000 TURNS VP=? NS= 100 TURNS VS= 5V - NP/NS =VP/VS  = 4000/100=VP/5  = 100VP= 20000  =VP= 200V THE ANSWER IS A = 100V – 240 V |  |  |
| Applicat ion | CLASS EXERCISE - A TV Transformer has 120 turns on its primary winding and 30 turns on its secondary coil. A 15volt A.C supply is connected to the primary winding (coil) of the transformer. - What will be the voltage across the secondary winding coil? ANSWER = 3.75 | Questions And Answers | Provide answers |
| Conclus ion | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CH

# KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:……

# BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS…

TOPIC:	TRANSFORMER	SUBTOPIC:	TRANSFORMATION RATIO, EMF EQUATION

# LEARNING OUTCOMES LSBAT

- Find the ratio of the given Transformer.

RATIONALE

Knowledge:	Basic calculations: Transformation ratio, Emf equation

Skills:	Calculation of transformation ratios

Values:	awareness of calculation or transformer ratio

TEACHING AIDS

| PART | LESSON CONTENT | METHOD OLOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| Introduc tion | - Recap on the previous lesson | Questions and answers | Provide answers |
| Develop ment | TRANSFORMATION RATIO, EMF EQUATION. BASIC CALCULATIONS EXAMPLE 3 A SESHEKE Sec School Transformer has a primary coil of 1000 turns and a secondary coil of 3000 turns. - What will be the voltage out of the secondary winding (core) if we apply 200v to the primary winding - Identify this type of transformer. SOLUTION Data - NP = 1000 TURNS VP= 200V NS= 3000TURNS VS=? A. - NP/NS =VP/VS  = 1000/3000 = 200/ VS  = 1000VS= 2000 X 3000 - VS= 600V ANSWER B. STEP UP TRANSFORMER EXAMPLE 4 A transformer has a primary voltage of 480 volts and the primary current of 2A. The secondary voltage of 240volts. Calculate the secondary current Data  VP= 480V IP= 2A IS= ? VS= 240 V NOTE - VP/VS = IS/IP Hence the voltage is step down from 480v to 240v while the  = 480/240=IS/2 current increase by a ratio of 1:2  = 240IS= 480 X 2 - IS= 4A ANSWER |  |  |
| Applicat ion | CLASS EXERCISE - A Kankoyo transformer has a primary winding core of 2000 turns and a secondary winding (coil) of 5000 turns - What is the voltage applied at the primary winding if the voltage out of the secondary winding is 600V? ANSWER = 240 V - Calculate the secondary current if the current at the primary is 60A? ANSWER= 24A | Questions And Answers | Provide answers |
| Conclus ion | - Recap on the main steps and points | V/E |  |

TEACHER:MWALE CH

## KENNETH KAUNDA SECONDARY SCHOOL DESIGN AND TECHNOLOGY DEPARTMENT

LESSON PLAN

SUBJECT: DESIGN AND TECHNOLOGY	CLASS:…… … DATE:……/……./20….	DURATION:………….

BOYS:…………………… GIRLS ………………………………………. TOTAL OF PUPILS… TOPIC:	TRANSFORMER	SUBTOPIC:   TRANSFORMER RATING LEARNING OUTCOMES LSBAT

- Find the rate of the given Transformer.

## RATIONALE

Knowledge:	Basic calculations: Transformation ratio, Emf equation

Skills:	Calculation of transformation ratios

Values:	awareness of calculation or transformer rating

## TEACHING AIDS

| PART | LESSON CONTENT | METHODO LOGY | LEARNER ACTIVITY |
| --- | --- | --- | --- |
| introduct ion | - Recap on the previous lesson | Questions and answers | Provide answers |
|  | TRANSFORMER TRANSFORMER RATING - Transformer are rated in Volts- Amperes (VA) or Kilovolts-Amperes (KVA) - This means that primary and secondary winding are designed to withstand the VA or KVA rating Stamped on the transformer name plate NOTE THAT - How to change KVA to VA  1000VA = 1 KVA FULL LOAD CURRENT - Equation For Full Load Current - Single Phase Transformer Va Rating Full Load Current = Voltage Or Kva X 1000 Full Load Current = Voltage - Three Phase Transformer Va Rating Full Load Current = 1.73 X Voltage Kva X 1000 Full Load Current = 1.73 X Voltage EXAMPLE 1. A single phase transformer with a 2 KVA rating has 480V primary and a 120 secondary. Determine - The primary full load current of the transformer - Secondary Full Load Current of the transformer SOLUTION Data  VP= 480V VS= 120 - Transformer Rating = 2kva = 2000va - Primary Full Load Current=(Va Rating)/(Voltage ) PFLC= 2000/480 PFLC= 4.17 A ANSWER - Secondary Full Load Current=(Va Rating)/(Voltage )  SFLC= 2000/120 - SFLC= 16.67A ANSWER |  |  |
| Applicati on | CLASS EXERCISE - A single phase transformer with a 2KVA rating has 240V on primary winding and a 60 V on secondary winding Determine - Primary full load currents of the transformer ANSWER PFLC= 8.33 A - Secondary full load currents of the transformer ANS=SFLC= 33.33A - What is the primary current of a transformer rated 3.75KVA and the primary voltage is 120V? ANSWER= 31.25 | Questions And Answers | Provide answers |
| Conclusi on | - Recap on the main steps and points | V/E |  |