CHINGOLA DISTRICT COMMMON EXAMINATION

Examination for the School Certificate (Grade 9) End of term 1- 2022

Design and Technology            6048/1

Theory and design                    Paper 1

CHINGOLA DISTRICT COMMMON EXAMINATION

Examination for the School Certificate (Grade 9) End of term 1- 2022

Design and Technology            6048/1

Theory and design                    Paper 1

Candidate Name________________________________________________

Candidates Name: …………………………………………………………………………… 155 Marks

Time: 2 hours   30 minutes

No Reading Time

Instructions to Candidates

- Write your name on the space provided for you on the cover page

- There are three (3) sections in this question paper, section A, B and C.

- Answer all the questions in this section A, and B on spaces provided,

- There are three parts in section C; part I, II and III. Questions 32 is compulsory, answer one question in each part.

- Write your answers in the space provided on the question paper

SECTION A  [30]

Multiple choice answer Grid.

| 1 |  |  |  | 2 |  |  |  | 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | A | B | C | D | A | B | C | D |
| 4 |  |  |  | 5 |  |  |  | 6 |  |  |  |
| A | B | C | D | A | B | C | D | A | B | C | D |
| 7 |  |  |  | 8 |  |  |  | 9 |  |  |  |
| A | B | C | D | A | B | C | D | A | B | C | D |
| 10 |  |  |  | 11 |  |  |  | 12 |  |  |  |
| A | B | C | D | A | B | C | D | A | B | C | D |
| 13 |  |  |  | 14 |  |  |  | 15 |  |  |  |
| A | B | C | D | A | B | C | D | A | B | C | D |
| 16 |  |  |  | 17 |  |  |  | 18 |  |  |  |
| A | B | C | D | A | B | C | D | A | B | C | D |
| 19 |  |  |  | 20 |  |  |  | 21 |  |  |  |
| A | B | C | D | A | B | C | D | A | B | C | D |
| 22 |  |  |  | 23 |  |  |  | 24 |  |  |  |
| A | B | C | D | A | B | C | D | A | B | C | D |
| 25 |  |  |  | 26 |  |  |  | 27 |  |  |  |
| A | B | C | D | A | B | C | D | A | B | C | D |
| 28 |  |  |  | 29 |  |  |  | 30 |  |  |  |
| A | B | C | D | A | B | C | D | A | B | C | D |

SECTION A  [30]

Multiple choice questions

Answer all questions. Mark your answers with a cross (X) on the answer Grid provided in the answer booklet.

- Mary was cutting by a piece of wood in the workshop. Suddenly, a piece of wood got into her eye. What would you do?

  - Clean the eye with water.

  - Cover her eyes with a wet cloth.

  - Report the matter to the teacher.

  - Take her to the clinic.

- When steel is coated with zinc to prevent rusting, it is said to have been

  - Alloyed.

  - Galvanized

  - Soldered.

  - Tempered.

- A design and technology teacher asked his pupils to define some plain figures. Which of the following statements describes a Hexagon?

  - Four sided figure.

  - Eight sided figure

  - Six sided figure

  - Three sided figure.

- Which of the following timber is classified as softwood?

  - Mukwa.                B. Elm.                  C.   Pine.                    D.   Teak

- In Zambia distribution system of electricity, the neutral wire is represented by ……

  - Blue.                    B.  Red.                  C.  Black.                  D.  Yellow.

- The following lines meet at 90° except……

  - Center lines.

  - Perpendiculars

  - Radius.

  - Tangents.

- What stage should the systematic investigation to establish the facts to be considered during the design process?

  - Analysis

  - Design brief.

  - Specification.

  - Research.

- The other name for bench clamp is …………..

  - Mitre box.

  - Bench stop.

  - Bench vice.

  - Bench hook.

- The following are manufactured boards, except ………….

  - Block board.

  - Chip board.

  - Urea formaldehyde.

  - Medium density fibre board.

- When a material can be stretched and be drawn into wire, it is said to be …….

  - Tough.

  - Ductile.

  - Malleable.

  - Brittle.

- The removal of excess moisture from timber is called ………..

  - Conversion

  - Felling

  - Seasoning

  - Sawing

- What is the name of the line indicated by the latter X as shown in the figure 1

below?

Fig. 1

  - Tangent.             B.  Diameter.              C. Normal.              D. Perpendicular.

- A state of being out of harm or danger …

  - Safety signs.

  - Protective.

  - First aid.

  - Safety.

- Which one of these is not a component of design and technology?

  - System technology.

  - Manufacturing materials.

  - Needle work.

  - Graphic communication.

- Bronze is an alloy of copper and ………..

  - Lead.                       B.   Zinc.                     C. Tin.                      D. Iron.

- ……………. are used for both cooling and heating depending on the air properties at a given time.

  - Vents.

  - Air extractors.

  - Fans.

  - Air conditioners.

- The drawing in figure 2 shows a tool used on the bench. study the drawing  and refer to it when  answering questions 17 and 18.

Figure 2

  - Bench well.

  - Bench hook.

  - Chopping board.

  - Bench stop.

- What is the use of the tool?

  - Chopping pieces on.

  - Glue parts on.

  - Supporting wood when cutting at 45.

  - Supporting work while sawing.

- When a formed thermoplastic is reheated, it returns to it’s original shape. This  is

called  ………….…………….

  - Deformation.

  - Elasticity.

  - Formation.

  - Plastics memory.

- At what stage of the design process, do you estimate the cost of the product?

  - Design brief.

  - Specification.

  - Possible solutions.

  - Evaluation.

- Plastics can be categorized into two groups, namely ………….

  - Ferrous and non-ferrous.

  - Thermo setting and a petroleum jelly.

  - Petroleum and thermoplastics.

  - Thermoplastic and thermosetting.

- The diagram below is showing two materials. One is made of wood and the other of acrylic plastic.

Figure 3

Name one type of glue that would be used to laminate them.

  - Polyvinyl  Acetate (PVA)

  - Animal glue

  - Varnish

  - Contact glue

- What is the best finish to apply on artifact that is made of mild steel which will outdoor?

  - Oil                  B. Varnish               C. Gloss paint              D. Water paint

- Most handles of cooking utensils are made of thermosetting plastics because it ………

  - Is a poor conductor of heat and electricity

  - Is hard, strong and easy to handle.

  - Resists corrosion and can easily be reshaped.

  - Resists rust and it is colorful.

- If your friend is injured in the workshop and he is bleeding, you should quickly  ………

  - Report to the teacher.

  - Stop the bleeding.

  - Take him to the clinic.

  - Wash the wound.

- In graphic communications, a pair of dividers is used for…….

  - Dividing a line into equal parts.

  - Dividing the angles into equal parts.

  - Transferring measurements from the drawing to the rule.

  - Transferring measurements from the rule to the drawing

- Another name for a bench hook is …………

  - Sawing board

  - Bench stop.

  - Mitre box.

  - Bench hold fast.

- Which of the labeled parts represents a chord?

Figure 4

- Which method would you use to improve the appearance of a drawing to make it more realistic?

  - Line shading

  - Punching

  - Rendering

  - Tone shading.

- Aluminum is used for making aeroplane bodies because it ………..

  - Cannot conduct electricity.

  - Is light in weight

  - Cannot conduct heat.

  - Has a good surface finish

SECTION B           [30 marks]

Answer all questions in the spaces provided in the answer booklet.

- a)    i.    Name the correct materials used in making the following products.

Aircraft body                                                                                [1]

Water pipes                                                                                 [1]

Musical instruments                                                                      [1]

ii.   The drawing in figure 8 shows a garden tool.

Figure 5

- Name a suitable material that can be used to make a garden tool.     [1]

- Give one property of the material you have mentioned in a (ii)         [1]

- What is the difference between thermosetting plastic and     thermoplastic                                                                                [2]

- What do you understand by the term plastic memory                        [1]

b)    The shape of the key holder is marked out as shown in figure 6.

Fig. 6

- What would you use to mark out the shape of the key holder?                        [1]

- Give a reason for your answer in (e), (i) above.                                             [1]

- What abrasive should be used to smoothen the edges of the key holder           [1]

c)    Figure 7 shows a cross section of a tree. Study it and answer the questions.

D

D

B

B

A

A

C

C

E

E

Fig. 7

Fig. 7

i         Label the parts marked  A, B and E                                                  [3]

ii.       State the function of the parts labelled C and D                              [2]

iii.      There are three elements that need to be in place for fire to take place;

State one of the three elements of fire.                                            [1]

iv.      List three items found in the first aid box.                                        [3]

(d)    Figure 7 below shows a tool that is used in Bricklaying

Fig. 8

- Name the tool in figure 7.                                                                     [1]

- State two materials that are aggregates                                              [2]

e) The drawing in figure 8 below shows an electric circuit in symbols.

fig. 9

- Label the components marked (i) and (ii)                                     [2]

- If the circuit has the current of 2A and the P.d of 6V, calculate its resistance.                                                                               [2]

- State the instrument used to measure

Resistance                                                                            [1]

Voltage                                                                                [1]

Current                                                                                [1]

[Total: 30 marks]

SECTION C       [95 marks]

There are three parts in this section part I, II, and III. Part I is compulsory.

Answer one question from part I and one question from part iii

Part I

Questions 32 on part I is Compulsory

- The following situation is obtaining regarding cooking utensils in your kitchen.

Figure 10

- (i)       Suggest a problem from the given situation.                                  [2]

(ii)      From the problem suggested in a (i) above, write a design brief      [3]

- From the design brief suggested, one of the specification to be considered.

- Indicate any other four (4) specifications to be considered.             [4]

- Sketch one possible solution you may consider.                              [10]

- What safety measures did you consider to make the possible

solution in (b)(ii) safe for use ?                                                     [2]

- (i)      Suggest any two types of materials you would use for your possible

solution.                                                                                     [2]

(ii) Give two reasons for your choice of material.                                 [2]

[Total: 25mark]

Part II Choose one question

- Either  (a) or (b)

- The home Economic Department would  like the  Design and Technology

section  to make some tea trays for them.

- Complete the cutting list that your teacher has started for you.

| Part | Qty | L | w | H | Material |
| --- | --- | --- | --- | --- | --- |
| Side rails |  | 500mm |  |  |  |
|  |  |  |  |  | Pine wood |
| Base | 1 |  |  | 18mm |  |

[12]

- Explain what is involved in the following stages of material preparation and the tools used.

- Measuring                                                                               [2]

- Marking out                                                                             [2]

- Sketch a free hand pictorial drawing of the tea tray. The sketch must be of good proportion, neat and include rendering.                               [5]

- Name a finish that could be applied on the tray and give the reasons for choosing that finish.                                                                      [4]

[Total: 25 marks]

- (i)     The drawing in figure 11 shows a garden  tap

Fig. 11

- Name one specific suitable materials that can be used to make a garden tap.                                                                                  [2]

- Give two properties of the material you have mentioned in b (1).      [2]

- List two examples of thermoplastics and thermosets.                       [4]

- Suggest the best ways of disposing off plastics.                               [2]

ii.   Figure below shows a tool used in the design and technology workshop

Fig, 12

- Name the tool                                                                              [2]

- State the function of the named tool above                                     [2]

- Name any marking out tool used to produce curves on plastics         [1]

- Draw a cutting tool to use when cutting a 4mm thick acrylic plastic.  [5]

- List (2) two types of sanding materials.                                         [2]

- Give three effects of plastic on the environment.                            [3]

[Total: 25 marks]

Part III

Graphic communication – Choose one question from 34 and 35

- A Design and technology pupil was told by the teacher to come up with designs of

making a tennis trophy for the school championships. ABC is a semi-ellipse. Draw the

tennis trophy full size showing all construction lines.

Fig. 12

[Total: 30 marks]

35         A new school has been built in Munani Community called Bwali Technical

School , and you being a student of Design and Technology has been asked

to Design a log of this school given data:

- Triangle ABC  which has the following measurements:

- AB=100mm

- AC=BC=80mm

a).    Construct the given triangle

b).    Inscribe a circle in triangle ABC.

c).    Circumscribe a circle to triangle ABC.

d).    Measure and state the diameter for the circumscribed circle.

e)     Print the name of the school on the segment below the triangle

[Total: 30 marks]